Avinashilingam Institute for Home Science and Higher Education for Women

(Deemed to be University under Category ‘A’ by MHRD, Estd. u/s 3 of UGC Act 1956)

Re-accredited with ‘A+’ Grade by NAAC. Recognised by UGC Under Section 12B

Coimbatore - 641 043, Tamil Nadu, India

Master’s Degree Examinations – November 2019

Semester –I / III

Class   :    I PG                                                           			Time  :3 hours

Major   :   Computer Science / Computer Applications 			 Max. Marks: 60

17MCSC04 / 17MCAC13 - DATA COMMUNICATION AND NETWORKS

CO1: Gain the knowledge of building different types of networks.

CO2: Identify and differentiate the functionalities on each layer in the protocol stack

CO3: Choose the different standards of Ethernet and suggest one for the organization based on their applications.

CO4: Able to develop different routing protocols for real world scenarios

CO5: Enhance the ideas gained to higher order communications systems.

PART A                   (10 * ½  = 5 Marks)

Choose the correct answer

| 1. |  | Actual communication occur through ___________medium |  | CO1K1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | - Transport b) Physical c) Interface d) Network |  |  |  |
| 2. | A port address in TCP/IP is ______ bits long. |  | CO2K1 |  |
|  | - 32 b) 64 c) 48 d) 16 |  |  |  |

| 3. | In _________, there is no resource allocation for a packet |  |  |  | CO2K1 |
| --- | --- | --- | --- | --- | --- |
|  | a) | circuit switching | b) | datagram switching |  |
|  | c) | frame switching | d) | packet-switching |  |

| 4. | Which multiplexing technique transmits digital signals? | CO3K2 |
| --- | --- | --- |
|  | - FDM. b) TDM. c) WDM d) None of the above |  |

5. Which error detection method uses one's complement arithmetic? CO3K1 a) Simple parity check b) Simple parity check c) Checksum. d) CRC

| 6. | _________codes are special linear block codes with one extra property. If a codeword is rotated, the result is another codeword. | CO3K2 |
| --- | --- | --- |
|  | - Non-linear b) Convolution. c) Cyclic d) Non cyclic. |  |

| 7. | Ethernet uses a __________ physical address that is imprinted on the network interface card (NIC). | CO3K1 |
| --- | --- | --- |
|  | - 32-bit. b) 64-bit c) 6-byte. d) 32-byte. |  |

| 8. | The Bluetooth radio layer performs functions similar to those in the Internet model's ___________ layer |  |  | CO5K2 |  |
| --- | --- | --- | --- | --- | --- |
|  | - Physical b) Data link c) Session d) Protocol |  |  |  |  |
| 9. | The __________ layer is responsible for the source-to-destination delivery of a packet across multiple network links. |  |  | CO5K1 |  |
|  | - transport b) network c) data link d) physical |  |  |  |  |
| 10. One of the responsibilities of the transport layer protocol is to create ________communication. CO5K1 - Host-to-host b) Process-to-process c) Node-to-node d) Interface-to-interface |  |  |  |  |
| PART – B (5 * 4 = 20 Marks) Answer All the Questions (Each answer should not exceed 200 words or one page) |  |  |  |  |
| # 11. | (a) | Analysis the various categories of topology | CO1K4 |  |
| (OR) |  |  |  |  |
| 11. | (b) | State the difference between a port address, a logical address, and a physical address | CO1K3 |  |
| # 12. | (a) | What is line coding? Discuss the Line coding and decoding in digital transmission | CO2K2 |  |
| (OR) |  |  |  |  |
| 12. | (b) | What is mean by switching? Explain briefly the circuit switching for data transmission | CO2K2 |  |
| # 13. | (a) | Analysis the concept of Single-Bit Error and Burst Error | CO3K4 |  |
| (OR) |  |  |  |  |
| 13. | (b) | Summarize the Sender site and Receiver site calculation for error detection using checksum method | CO5K5 |  |
| # 14. | (a) | Describe the IEEE standard for LANs with diagram | CO4K2 |  |
| (OR) |  |  |  |  |
| 14. | (b) | List out the Bluetooth layers and its functions | CO5K2 |  |
| # 15. | (a) | Draw the format of an ARP packet and explain the each field | CO5K2 |  |
| (OR) |  |  |  |  |
| 15. | (b) | Discuss the responsibilities of the transport layer in the Internet model | CO4K2 |  |
| PART – C (5 *7 = 35 Marks) Answer All the Questions (Each answer should not exceed 600 words or three page) |  |  |  |  |
| # 16. | (a) | List out the layers in OSI reference Model and explain its functions | CO1K2 |  |
| (OR) |  |  |  |  |
| 16. | (b) | Explain the relationship of layers and addresses in TCPIIP | CO1K3 |  |
| # 17. | (a) | What is Guided Media Transmission? Explain the basics of guided transmission medium | CO2K2 |  |
| (OR) |  |  |  |  |
| 17. | (b) | Compare the features of ADSL, VDSL, HDSL, and SDSL DSL technology | CO2K3 |  |
| # 18. | (a) | Justify the performance of Simple Parity-Check Code with example | CO2K4 |  |
| (OR) |  |  |  |  |
| 18. | (b) | Compare and contrast the CSMA and CSMA/CD Random Access protocol with various aspects | CO4K4 |  |
| # 19. | (a) | Discuss the architecture and MAC sublayer of IEEE 802.11 standard. | CO4K2 |  |
| (OR) |  |  |  |  |
| 19. | (b) | Summarize the functions of the various connecting devices | CO4K4 |  |
| # 20. | (a) | List three forwarding techniques and give a brief description of each. | CO5K2 |  |
| (OR) |  |  |  |  |
| 20. | (b) | Illustrates the process-to-process delivery by the transport layer. | CO5K3 |  |
| ************ |  |  |  |  |

SCHEME OF VALUATION

DATA COMMUNICATION AND NETWORKS

Sub Code: 17MCSC04 / 17MCAC13

SET -1

PART A (10 * ½  = 5 Marks)

Circle the correct answer

1.b). Physical

2.a). 32

3. b). Datagram switching

4.b). TDM

5.c). Checksum

6.c). Cyclic

7.c). 6-byte

8.a). Physical

9.b). Network

10.b). Process-to-process

| PART – B (5 * 4 = 20 Marks) Answer All the Questions (Each answer should not exceed 200 words or one page) |  |  |  |
| --- | --- | --- | --- |
| # 11. | (a) | Categories of topology: Mesh, star, Bus, Rind – analysis – explanation – 4 Marks |  |
| (OR) |  |  |  |
| 11. | (b) | Difference between a port address, a logical address, and a physical Address – explanations – 4 Marks |  |
| # 12. | (a) | Line coding is the process of converting digital data to digital signals. -Line coding and decoding - Signal Element Versus Data Element-Data Rate Versus Signal Rate – explanation– 4 Marks |  |
| (OR) |  |  |  |
| 12. | (b) | Physical path all the way carrier switching office-physical connection is established- explanation with diagram -4 Marks |  |
| # 13. | (a) | Whenever bits flow from one point to another, they are subject to unpredictable changes because of interference. This interference can change the shape of the signal- Types of Errors: Single-Bit Error- Burst Error – explanations – 4 Marks |  |
| (OR) |  |  |  |
| 13. | (b) | Sender site: 1. The message is divided into 16-bit words. 2. The value of the checksum word is set to O. 3. All words including the checksum are added using one's complement addition. 4. The sum is complemented and becomes the checksum. 5. The checksum is sent with the data. Receiver site: 1. The message (including checksum) is divided into 16-bit words. 2. All words are added using one's complement addition. 3. The sum is complemented and becomes the new checksum. 4. If the value of checksum is 0, the message is accepted; otherwise, it is rejected.– 4 Marks |  |
| # 14. | (a) | IEEE standard for LANs -LLC: Logical link control - LLC: Logical link control - OSI or Internet model – HDLC frame compared with LLC and MAC frames – explanations – 4 Marks |  |
| (OR) |  |  |  |
| 14. | (b) | Bluetooth Layers : Bluetooth uses several layers that do not exactly match those of the Internet model- Radio Layer, Baseband Layer- Physical Links – explanations – 4 Marks |  |
| # 15. | (a) | Draw the format of an ARP packet – Fields: Hardware type, Protocol type, Hardware length, Protocol length, Operation, Sender hardware address, Sender protocol address, Target hardware address – explanations – 4 Marks |  |
| (OR) |  |  |  |
| 15. | (b) | Responsible for process-to-process delivery - responsible for the delivery of a message from one process to another - Service-point addressing - Segmentation and reassembly - Connection control - Flow control – explanations– 4 Marks |  |

| PART – C (5 *7 = 35 Marks) Answer All the Questions (Each answer should not exceed 600 words or three page) |  |  |  |
| --- | --- | --- | --- |
| # 16. | (a) | OSI reference models- Physical layer, Data Link layer, Network layer, Transport layer, Session layer, Presentation layer, Application layer- Explanations with Diagram – 7 Marks |  |
| (OR) |  |  |  |
| 16. | (b) | Four levels of addresses are used in an internet employing the TCP/IP protocols - physical (link) addresses, logical (IP) addresses, port addresses, and specific addresses - relations ships explanations – 7 Marks |  |
| # 17. | (a) | Guided Media: These are those that provide a conduit from one device to another that include twisted-pair, coaxial cable and fiber-optic cable. A signal traveling along any of these media is directed and is contained by the physical limits of the medium. Twisted-pair and coaxial cable use metallic that accept and transport signals in the form of electrical current –Explanations - 7 Marks |  |
| (OR) |  |  |  |
| 17. | (b) | Features of ADSL, VDSL, HDSL, and SDSL DSL technology – explanations – 7 Marks |  |
| # 18. | (a) | A simple parity-check code is a single-bit error-detecting code in which n =k + 1 with dmin=2 - example - Simple parity-check code C(5, 4) – explanations |  |
| (OR) |  |  |  |
| 18. | (b) | Transmission is random among the stations- no rules specify which station should send next. Stations compete with one another to access the medium- CSMA and CSMA/CD Random Access protocol – differentiate – explanations – 7 Marks |  |
| # 19. | (a) | Architecture: The standard defines two kinds of services: the basic service set (BSS) and the extended service set (ESS) – explanations MAC sublayer of IEEE 802.11 : two MAC sublayers - relationship between the two MAC sublayers – explanations – 7 Marks |  |
| (OR) |  |  |  |
| 19. | (b) | Connecting devices : Passive Hubs- Repeaters - Active Hubs- Bridges- Two-Layer Switches - Three-Layer Switches – Gateway – functions of each connecting devices – explanations – 7 Marks |  |
| # 20. | (a) | Forwarding means to place the packet in its route to its destination. Forwarding requires a host or a router to have a routing table. Forwarding Techniques: Next-Hop Method Versus Route Method - Network-Specific Method Versus Host-Specific Method- Default Method – explanations – 7 Marks |  |
| (OR) |  |  |  |
| 20. | (b) | Client/Server Paradigm - IP addresses versus port numbers - Socket Addresses - Multiplexing and Demultiplexing - explanations– 7 Marks |  |
| ************* |  |  |  |