Avinashilingam Institute for Home Science and Higher Education for Women

(Deemed to be University under Category ‘A’ by MHRD, Estd. u/s 3 of UGC Act 1956)

Re-accredited with ‘A+’ Grade by NAAC. Recognised by UGC Under Section 12B

Coimbatore - 641 043, Tamil Nadu, India

Master’s Degree Examination – November 2019

III Semester

Class   :  II PG                                                           			         Time  :3 hours

Major :  Computer Application					          Max. Marks: 60

17MCAC13 - DATA COMMUNICATION AND NETWORKS

.

PART A                                                 (10 x  ½  = 5)

Choose the correct answer

| 1. | The OSI model consists of _________ layers |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) | three | b) | five |  |
|  | c) | eight | d) | seven |  |

| 2. | ________is the protocol suite for the current Internet. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) | OSI | b) | TCP/IP |  |
|  | c) | ARPANET | d) | NCP |  |

| 3. | Circuit switching takes place at the ________ layer |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) | data link | b) | physical. |  |
|  | c) | transport. | d) | network. |  |

| 4. | Transmission media are usually categorized as _________. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) | fixed or unfixed. | b) | guided or unguided. |  |
|  | c) | determinate or indeterminate. | d) | metallic or nonmetallic. |  |

| 5. | __________ is a process-to-process protocol that adds only port addresses, checksum error control, and length information to the data from the upper layer |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) | TCP | b) | IP |  |
|  | c) | UDP | d) | ARP |  |

| 6. | In cyclic redundancy checking, what is the CRC? |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) | The divisor. | b) | The quotient. |  |
|  | c) | The remainder. | d) | The dividend. |  |

| 7. | The ___________ is responsible for delivering data units from one station to the next without errors |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) | Network layer | b) | Transport layer |  |
|  | c) | Data link layer | d) | Session layer |  |

| 8. | The wireless LAN access method is ___________ |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) | CSMA/CA | b) | TDMA |  |
|  | c) | FDMA | d) | None |  |

| 9. | One of the responsibilities of the transport layer protocol is to create a ________communication. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) | Host-to-host | b) | Process-to-process |  |
|  | c) | Node-to-node | d) | Interface-tointerface |  |

| 10. | Mapping of a logical address to a physical address can be __________ |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | a) | Static | b) | Dynamic |  |
|  | c) | Static or dynamic | d) | Resolution protocol |  |

| PART – B (5 x 4 = 20) Answer All the Questions Each answer should not exceed 200 words or one page |  |  |  |
| --- | --- | --- | --- |
| # 11. | (a) | Name the four basic network topologies, and cite an advantage of each type. |  |
| (OR) |  |  |  |
| 11. | (b) | Compare the TCP/IP and OSI reference model. |  |
| # 12. | (a) | Describe the analog and digital signals with diagram. |  |
| (OR) |  |  |  |
| 12. | (b) | Discuss the significance of Unguided media. |  |
| # 13. | (a) | Explain the process of error detection in block coding. |  |
| (OR) |  |  |  |
| 13. | (b) | What is Cyclic Codes? Explain the Cyclic Redundancy Check. |  |
| # 14. | (a) | Draw the 802.3 MAC frame format and explain. |  |
| (OR) |  |  |  |
| 14. | (b) | Compare the Bus Backbone and Star Backbone topology. |  |
| # 15. | (a) | Analyze the protocols which create a mapping between physical and logical addresses. |  |
| (OR) |  |  |  |
| 15. | (b) | What is an error-reporting message? Explain. |  |

| PART – C (5 x 7 = 35) Answer All the Questions Each answer should not exceed 600 words or three page |  |  |  |
| --- | --- | --- | --- |
| # 16. | (a) | Explain the layer in OSI Network model with neat diagram. |  |
| (OR) |  |  |  |
| 16. | (b) | Explain the relationship of Layers in the TCP/IP protocol suite. |  |
| # 17. | (a) | Describe the digital data transmission methods in detail. |  |
| (OR) |  |  |  |
| 17. | (b) | Demonstrate the Frequency-Division Multiplexing and Demultiplexing Process with diagram. |  |
| # 18. | (a) | Compare the CRC encoder and decoder with diagram. |  |
| (OR) |  |  |  |
| 18. | (b) | Compare and contrast the Noiseless Channels protocols in data link control. |  |
| # 19. | (a) | Explain the Bluetooth architecture and layers. |  |
| (OR) |  |  |  |
| 19. | (b) | List out the five categories of connecting devices and compare the features of connecting devices. |  |
| # 20. | (a) | List three Popular routing protocols and give a brief description of Distance Vector Routing. |  |
| (OR) |  |  |  |
| 20. | (b) | Compare and contrast the open and closed loop congestion control. |  |
| ************* |  |  |  |

SET -2

SCHEME OF VALUATIONS

DATA COMMUNICATION AND NETWORKS

Sub Code: 17MCSC04 / 17MCAC13

PART A (10 * ½  = 5 Marks)

Circle the correct answer

1.d). Seven

2.b). TCP/IP

3.b). Physical

4.b). Guided or unguided.

5.c). UDP

6.c). The remainder

7.c). Data link layer

8.a). CSMA/CA

9.b). Process-to-process

10.c).static or dynamic

| PART – B (5 * 4 = 20 Marks) Answer All the Questions (Each answer should not exceed 200 words or one page) |  |  |  |
| --- | --- | --- | --- |
| # 11. | (a) | Mesh, star, Bus, Rind – advantage of each type – explanation- 4 Marks |  |
| (OR) |  |  |  |
| 11. | (b) | TCP/IP protocol suite is made of five layers: physical, data link, network, transport, and application- OSI have seven layers- functions of layers- explanations - 4 Marks |  |
| # 12. | (a) | Signals can be analog or digital. Analog signals can have an infinite number of values in a range; digital signals can have only a limited number of values - diagram Data can be analog or digital. Analog data are continuous and take continuous values. Digital data have discrete states and take discrete values – explanations - 4 Marks |  |
| (OR) |  |  |  |
| 12. | (b) | Unguided media transport electromagnetic waves without using a physical conductor. This type of communication is often referred to as wireless communication- wireless communication, ground propagation, sky propagation, and line-of-sight propagation - diagram- 4 Marks |  |
| # 13. | (a) | Process of error detection in block coding - divide message into blocks, each of k bits, called data words - The resulting n-bit blocks are called code words - sender creates code words out of data words by using a generator that applies the rules and procedures of encoding – diagram – explanations - 4 Marks |  |
| (OR) |  |  |  |
| 13. | (b) | Cyclic codes are special linear block codes with one extra property. In a cyclic code, if a code word is cyclically shifted (rotated), the result is another code word - Cyclic Redundancy Check - create cyclic codes to correct errors- used in networks LANs and WANs. - A CRC code with C(7, 4) – explanations - 4 Marks |  |
| # 14. | (a) | The Ethernet frame contains seven fields: preamble, SFD, DA, SA, length or type of protocol data unit (PDU), upper-layer data, and the CRC- frame format – explanations - 4 Marks |  |
| (OR) |  |  |  |
| 14. | (b) | In a bus backbone, the topology of the backbone is a bus. - Bus backbone diagram In a star backbone, the topology of the backbone is a star; the backbone is just one switch– explanations – 4 Marks |  |
| # 15. | (a) | An internet is made of a combination of physical networks connected by internetworking devices such as routers- Mapping Logical to Physical Address - ARP operation- Cache Memory - Packet Format -explanations – 4 Marks |  |
| (OR) |  |  |  |
| 15. | (b) | The error-reporting messages report problems that a router or a host (destination) may encounter when it processes an IP packet - Message Format- ICMP always reports error messages to the original source. - Error-reporting messages - ICMP error messages - Contents of data field for the error messages – explanations – 4 Marks |  |

| PART – C (5 *7 = 35 Marks) Answer All the Questions (Each answer should not exceed 600 words or three page) |  |  |  |
| --- | --- | --- | --- |
| # 16. | (a) | Network architecture - network designers have developed general blueprints—usually called a network architecture - OSI reference models- Physical layer, Data Link layer, Network layer, Transport layer, Session layer, Presentation layer, Application layer-Explanations- Diagram 7- Marks |  |
| (OR) |  |  |  |
| 16. | (b) | Layers in the TCP/IP protocol suite do not exactly match those in the OSI model – difference in layer and function - five layered TCP/IP protocol suit – Relationship of layers in TCPIIP explanations - 7 - Marks |  |
| # 17. | (a) | Digital Transmission- Digital To Digital Conversion: Line Coding- Line Coding Schemes - Block Coding – Scrambling – explanations -7 Marks |  |
| (OR) |  |  |  |
| 17. | (b) | Frequency-division multiplexing (FDM) is an analog technique that can be applied when the bandwidth of a link (in hertz) is greater than the combined bandwidths of the signals to be transmitted – Multiplexing Process - FDM Demultiplexing Process - diagram – explanations -7 Marks |  |
| # 18. | (a) | Comparisons - CRC encoder and decoder- Division in CRC encoder- Division in the CRC decoder for two cases – example – diagram – explanations -7 Marks |  |
| (OR) |  |  |  |
| 18. | (b) | Two protocols for this type of channel. The first is a protocol that does not use flow control; the second is the one that does - Compare and contrast with desing , algorithm, sender and receiver – explanations -7 Marks |  |
| # 19. | (a) | Bluetooth is a wireless LAN technology designed to connect devices of different functions such as telephones, notebooks, computers, cameras, printers Bluetooth defines two types of networks: piconet and scatternet – explanations Bluetooth Layers : Bluetooth uses several layers that do not exactly match those of the Internet model- Radio Layer, Baseband Layer- Physical Links – explanations – 7 Marks |  |
| (OR) |  |  |  |
| 19. | (b) | Divide connecting devices into five different categories based on the layer in which they operate in a network - Passive Hubs- Repeaters- Active Hubs- Bridges – compare the features of connecting devices -explanations -7 Marks |  |
| # 20. | (a) | List three Popular routing protocols – Link state – path vector Distance Vector Routing : Distance vector routing tables- Initialization of tables in distance vector routing – Sharing - Diagram – explanations |  |
| (OR) |  |  |  |
| 20. | (b) | Congestion control: Congestion in a network may occur if the load on the network-the number of Packets sent to the network-is greater than the capacity of the network-the number of packets a network can handle –Open loop congestion control - Closed-Loop Congestion Control - explanations– 7 Marks |  |
| ************* |  |  |  |